Minutes of DRAGON Meeting of March 22, 2005

Present: AL, DG, CR, MT, JD'A (recorder replacement), LB, GR, PM, DH, AO,

guests from York, JC, CV, DFOT,  JP, BG

- Minutes of 03/15/05 meeting reviewed and approved with no changes.

- Business arising:

Action item: Red Flag now indicates ED1 off

- Comments from DRAGON Operations in Ne Run

  - Charge X-slit may not be moving properly as it was noticed that it was going slowly to new positions and perhaps not getting to new positions. It may be a matter of the method of control system but will be checked again. The proper operation is to do Calibration is not sure.

  - Connection on water flow for cooling TP on gas target broke off leading to a flood. It has been fixed.

  - Major blackout during run at ~3am on Sunday. MT was able to get all units back on but there was a problem with undocumented circuit breaker on TP control systems. DFOT get it going (with a comb) but this need some type of documentation on web page 	ACTION item

  - FCM bias goes off when closed and it would be good if it was turned on automatically when FCM is inserted. The same can be said for FCCH. This needs some attention in EPICS system by RB   ACTION item

  - Pumping on cryopump for ED2 very slow. DFOT mentioned that there is a serious leak in turbo connection piping and needs fixing..ACTION item

  - Apparently the various displays for different device operations sometime appear on the computer behind other displays and hard to know. JP will fix this part of the video system. 			ACTION item

- Results from DRAGON 20Ne(p,gamma) study

  - From the prospective of operations, the run was successful as all experimental objectives were achieved and data obtained. There were some tuning problems at times but energy changes did go somewhat smoothly. Beam energies disagree with Prague magnet is a different way now. Slits needed to be reinstalled at Prague magnet.

  - An initial quick analysis of the results do indicate some possible problems with some aspects of the data, e.g. energy of the resonance is off from literature value, and the observed width of resonance may be off slightly. A more detailed and careful analysis of all data needed especially given possible problem with Charge Slit as mentioned above.

- Future Runs

a. CV presented a tentative outline of a game plan for the first 40Ca++ run which will probably happen in either 10 days or later depending upon on stable beam schedule. See attached for outline and some related graphs.

Meeting adjourned at 11:35 am